| ## Report Description | M2GL05 CORE & I/O Common Circuit Block |
| --- | --- |
| ## Report No | M2GL05-CCB-001 |
| ## Issue No | Revision 00a |
| ## Report Date | 21/11/2025 |
| ## Report State | Rev00a |

## Summary

This report defines a reusable Core & I/O Common Circuit Block (CCB) based on the Microchip IGLOO2 M2GL005 FPGA. The block provides a flexible, low-power, secure, and highly reliable foundation for digital control, monitoring and debug applications. It has been designed with a strong emphasis on signal integrity, power-rail stability, modular I/O allocation and long-term reliability, making it particularly suitable for integration into safety-focused and high-noise development environments such as power electronics, motor control and aerospace test systems. Disclaimer / Usage Notice This Common Circuit Block (CCB) is provided for engineering, educational and development purposes only. It is intended as a reusable FPGA core and I/O platform and does not, by itself, constitute a complete or certified system. Correct and safe operation is highly dependent on PCB layout, assembly quality, protection design, enclosure, and overall system integration. No liability is accepted for any injury, equipment damage, financial loss, or consequential damage arising from the use or misuse of this design, whether in whole or in part. The responsibility for verification, certification and compliance with relevant standards remains with the integrator.

Open Source Hardware License Statement — CERN-OHL-S v2 (Strongly Reciprocal)

This document, together with the hardware design concepts, architecture, descriptions, and associated technical material contained herein, is released under the CERN Open Hardware Licence Version 2 – Strongly Reciprocal (CERN-OHL-S v2).

You are permitted to use, study, modify, manufacture, and distribute products based on this material, provided that:

- Attribution to the original author is preserved:
Andy Carpenter — MBSTech

- Any redistribution or derivative work is released under the same licence (CERN-OHL-S v2)

- The source of any modifications is clearly documented and made available

This work is provided “as is”, without warranty of any kind, express or implied, including but not limited to fitness for purpose, safety, merchantability, or non-infringement.

The author(s) and contributors shall not be held liable for any direct, indirect, incidental, or consequential damages arising from the use, misuse, or implementation of this work, whether in part or in full.

The responsibility for meeting all legal, safety, EMC, insulation, and certification requirements in any final implementation rests entirely with the user or integrator of this design.

The full licence text may be obtained at:
CERN Open Hardware Licence Version 2 – Strongly Reciprocal (CERN-OHL-S v2)

Contents

Report Description	1

Report No	1

Issue No	1

Report Date	1

Report State	1

Summary	1

References	4

Abbreviations	5

Purpose and Problem Statement	6

Scope	6

Introduction of CCB	6

Block Diagram	7

Intended Applications	7

Out of Scope / Non-Goals	7

Typical Use Case Scenarios	8

Key Design Drivers (KDDs)	9

Design Architecture & Philosophy	9

Practical Implementation Notes (PCB Focused)	10

Test / Bring-Up Procedure	11

Electrical Specification Summary	12

## References

1:  IGLOO2 FPGA M2GL005 Datasheet
https://ww1.microchip.com/downloads/aemDocuments/documents/FPGA/ProductDocuments/DataSheets/IGLOO-2-FPGA-And-SmartFusion-2-SoC-FPGA-Data-Sheet-DS00004750.pdf

2: Microchip – IGLOO2 Clocking Resources & PLL User Guide
Microchip IGLOO2 Clocking Guide (navigate “Documentation→IGLOO2→Clocking Resources”)

3: Texas Instruments – CDC6CE025 Clock Generator Datasheet (CDC6CE025000ADLER)
TI CDC6CE025 Datasheet

https://www.ti.com/lit/gpn/cdc6c

4: Integrated Silicon Solution Inc. – IS66WVQ8M4DBLL-133BLI Pseudo-SRAM Datasheet
https://www.issi.com/WW/pdf/66-67WVQ8M4DALL-BLL.pdf

5: Murata Manufacturing Co. – BLM15AG601SN1D Ferrite Bead Datasheet
Murata BLM15AG601SN1D

https://www.mouser.co.uk/datasheet/3/76/1/ENFA0018.pdf

6: Microchip – AN4153: IGLOO2 Power Supply Decoupling & Layout Guidelines
Microchip AN4153 Application Note

https://ww1.microchip.com/downloads/aemDocuments/documents/FPGA/ApplicationNotes/ApplicationNotes/Microchip_SmartFusion2_and_IGLOO2_Board_and_Layout_Design_Guidelines_AN4153_VC.pdf

7: Institute of Electrical and Electronics Engineers (IEEE) – IEEE 1149.1 JTAG® Test Access Port and Boundary-Scan Architecture
IEEE 1149.1 Standard Overview

8: Texas Instruments – TPSM828438VCFR Step-Down Power Module Datasheet – https://www.ti.com/product/TPSM82843/part-details/TPSM828438VCFR

9: Texas Instruments – TPSM828437VCFR Step-Down Power Module Datasheet – https://www.ti.com/product/TPSM82843/part-details/TPSM828437VCFR

## Abbreviations

| Abbreviation | Meaning |
| --- | --- |
| CCB | Common Circuit Block |
| FPGA | Field Programmable Gate Array |
| IGLOO2 | Microchip IGLOO2 Flash-based FPGA family |
| SRAM | Static Random Access Memory |
| QSPI | Quad Serial Peripheral Interface |
| JTAG | Joint Test Action Group (IEEE 1149.1) |
| PLL | Phase-Locked Loop |
| VDDA | Analogue supply voltage for PLL circuitry |
| VSSA | Analogue ground reference for PLL circuitry |
| SSR | Series Source Resistor |
| FTDI | Future Technology Devices International |
| BOM | Bill of Materials |
| MTBF | Mean Time Between Failures |
| FIT | Failures In Time (failures per 10^9 hours) |
| EMI | Electromagnetic Interference |
| ESD | Electrostatic Discharge |
| MSIO | Multi-Standard Input/Output |
| DDRIO | Double Data Rate Input/Output |
| UUT | Unit Under Test |
| POD | Debug or Programming Onboard Device |

## Purpose and Problem Statement

Modern embedded systems require a flexible, reliable and secure digital core capable of interfacing with a wide range of external devices and communication standards. In safety-critical, high-voltage, or electrically hostile environments, additional requirements such as deterministic startup, resistance to transient disturbances, and robust long-term reliability are essential.

The M2GL05 CORE & I/O CCB addresses these challenges by providing a compact, flash-based FPGA platform with carefully engineered power, clocking and decoupling infrastructure. The platform is intended to serve as a reusable digital backbone for control, monitoring, communications and debug functions within modular architectures such as the Debug POD ecosystem.

## Scope

This Common Circuit Block (CCB) defines a modular, FPGA-based core and I/O platform intended to provide a flexible digital processing, control, and interfacing foundation for larger electronic systems. The scope of this block includes the provisioning of the IGLOO2 (M2GL005) FPGA, supporting power rails, clocking infrastructure, decoupling, configuration interfaces, and optional external SRAM expansion required for stable and reliable operation.

The CCB is responsible for supplying a robust, well-controlled internal environment for the FPGA, including clean and regulated operating voltages, low-jitter clock input, and correctly biased configuration and JTAG interfaces. It provides a structured and reusable foundation by which higher-level, application-specific functions may be implemented within the FPGA fabric without requiring repeated redesign of the underlying support circuitry.

This block is intentionally designed to be protocol- and application-agnostic. While it is optimised to integrate with associated Common Circuit Blocks such as the Isolated USB Interface and Debug POD architecture, it may also be deployed as a standalone FPGA platform for a wide range of digital control, signal-processing, communications, or instrumentation roles.

The scope of this design does not include any application-specific signal conditioning, analogue interfacing, high-voltage power electronics, sensing elements, or actuators. These elements are to be implemented in separate, external circuit blocks that connect via the general-purpose I/O resources and defined interfaces of this CCB.

## Introduction of CCB

The heart of this CCB is the Microchip IGLOO2 M2GL005 FPGA in a 256-pin VFBGA package. This device is a low-power, flash-based FPGA offering non-volatile configuration, deterministic power-up and enhanced security features. Surrounding circuitry provides stable power rails (P1V2 core, P2V5, and P3V3), comprehensive local decoupling, precision clocking, and optional external SRAM expansion for enhanced buffering and data-capture capability.

All general-purpose I/O resources are intentionally left uncommitted in this design. This allows the block to be freely adapted for a wide range of future applications, protocols, and hardware interfaces without requiring fundamental changes to the underlying core architecture.

## Block Diagram

Figure 1 Basic Block Diagram of CCB

## Intended Applications

This Common Circuit Block (CCB) is intended for use as a flexible, FPGA-based digital core within a wide range of embedded systems and development platforms. It provides a stable and highly configurable processing and I/O foundation for applications requiring deterministic behaviour, low power consumption, and long-term reliability.

Primary intended applications include, but are not limited to:

• Digital control platforms for power-electronics converters, motor-control systems, and embedded controllers
• High-speed debug and monitoring engines within the Debug POD architecture
• Protocol translation and bridging (e.g. SPI, UART, custom synchronous or Manchester-type links)
• Data acquisition pre-processing and buffering using internal FPGA resources and optional external SRAM
• Laboratory instrumentation, test fixtures, and development backplanes
• Aerospace, automotive or industrial sub-systems requiring deterministic start-up and radiation-tolerant technology
• Custom communication controllers for isolated or high-noise environments when paired with other CCBs

The intentionally uncommitted nature of the general-purpose I/O makes this block well suited for future expansion and reconfiguration, allowing it to adapt to evolving system requirements without the need for full hardware redesign.

## Out of Scope / Non-Goals

This Common Circuit Block (CCB) is not intended to implement any application-specific functional behaviour by default. It does not include predefined control algorithms, closed-loop regulation schemes, motor-control firmware, power-conversion logic, sensor conditioning circuitry, or safety-critical interlock functions. These elements are expected to be developed within the FPGA fabric or implemented in separate, application-specific blocks as required by the end system.

The CCB is also not designed to act as a general-purpose computing platform or operating-system host. It is not intended to replace a microprocessor- or microcontroller-based architecture where complex operating systems, user interfaces, or high-level networking stacks are required.

Whilst the design includes optional external SRAM to support buffering and high-speed data capture, this block is not intended to provide large-scale non-volatile storage, mass data logging, or file-system-based memory functions. Any system requiring significant data retention must implement dedicated external memory solutions beyond the scope of this CCB.

The M2GL05 CCB does not claim compliance with any specific regulatory approval (e.g. CE, UL, IEC, MIL) as a standalone entity. Certification, environmental compliance, safety approvals, and final mechanical safeguarding remain entirely the responsibility of the integrator incorporating this block into a complete product or system.

Integration Guidelines with Other CCBs

This Core & I/O CCB is intended to operate as part of a larger modular ecosystem composed of multiple Common Circuit Blocks, each responsible for a defined subset of system functionality. When integrated with other CCBs, strict attention shall be paid to domain separation, signal integrity, and power distribution in order to preserve both performance and safety intent.

When used in conjunction with the Isolated USB Interface CCB, all programming, debug and data interfaces (JTAG, SPI, UART, or custom high-speed links) shall be routed exclusively through the isolated secondary domain of that module. No direct electrical connection shall be made between the USB-host reference (0V_USB) and the M2GL05 local ground (0V), as this would defeat the isolation strategy and introduce unsafe ground loops.

If the M2GL05 CCB is used as the central logic element within the Debug POD architecture, it shall be powered solely from the isolated secondary supply (nominally P3V3 and P1V2) generated by the supporting CCBs. Under no circumstances should power be back-fed into this block from both isolated and non-isolated sources simultaneously.

When integrating optional Personality Modules or interface cards, these shall be connected through defined header interfaces that respect the M2GL05 I/O bank voltage limits, current capability, and signal standards. All external modules must share the same local reference (0V) and shall not introduce alternate ground references or hidden conductive paths across isolation boundaries.

Where high-speed links or synchronous clocked interfaces are employed between CCBs, controlled-impedance routing, cable shielding (where applicable), and clean return paths must be maintained. Particular attention shall be paid to limiting stub lengths and avoiding via stacks or unnecessary connectors in timing-critical paths.

This CCB is designed to function as a digital core within a layered architecture. Other functional blocks (analogue front ends, power stages, sensors, actuators, etc.) are expected to interface through well-defined protocol or data boundaries, maintaining clear functional and electrical separation between subsystems.

## Typical Use Case Scenarios

Isolated Debug and Programming Platform
The M2GL05 Core & I/O CCB is used as the central logic engine within an isolated Debug POD. In this configuration, it receives programming, debug and data commands via the Isolated USB CCB and executes JTAG, SPI, UART or custom high-speed communication with a high-voltage or electrically noisy unit-under-test. The FPGA may implement protocol translation, data capture, timing analysis, and event monitoring without exposing the host computer or user to hazardous potentials.

High-Speed Data Acquisition Node
The block can be deployed as a local data pre-processor in a distributed measurement system. Sensor and control signals are routed into the FPGA via uncommitted I/O banks, where real-time processing, filtering, and buffering (including use of external SRAM) are performed before forwarding compacted datasets to a host system or storage device via a secondary communications CCB.

Custom Control Engine for Power Electronics
Within power-conversion or motor-control prototypes, the M2GL05 CCB can act as a deterministic, low-latency control engine. Timing-critical tasks such as PWM generation, phase measurement, synchronisation, and protection logic can be implemented in hardware, while still allowing safe external reconfiguration and monitoring during development.

Protocol Bridge / Translator
The FPGA may be configured to act as a real-time bridge between dissimilar communication standards (e.g. legacy single-ended interfaces and modern LVDS or Manchester-encoded links). This enables gradual system upgrades without requiring immediate replacement of all upstream or downstream components.

Aerospace or High-Reliability Subsystem
Owing to the flash-based, non-volatile configuration and radiation-tolerant characteristics of the IGLOO2 architecture, this CCB may be deployed in high-reliability environments such as aerospace ground equipment, laboratory instrumentation, or long-life industrial controllers where deterministic start-up and configuration permanence are required.

In all scenarios, the same underlying principle applies: the M2GL05 CCB provides a safe, stable, and highly configurable digital core that can be rapidly adapted to new roles without modification to the underlying power, clocking, or configuration infrastructure.

## Key Design Drivers (KDDs)

- Low-power, flash-based FPGA technology (IGLOO2)

- Deterministic, non-volatile startup behaviour

- Modular and uncommitted I/O allocation

- Support for high-speed debug and data streaming

- Robust power integrity and decoupling strategy

- Long-term reliability (25-year design intent)

- Compatibility with Debug POD and future CCB modules

- Reduced external component count where practical

- Provision for optional external SRAM expansion

- EMI/ESD-conscious design and filtering

## Design Architecture & Philosophy

The design follows a distributed decoupling philosophy, with local 100 nF capacitors placed across all major supply rails (P1V2, P2V5 and P3V3) and additional bulk capacitance on the P1V2 core rail to support transient current demands. This approach ensures low supply impedance across a wide frequency range and improves FPGA stability under dynamic loading conditions.

A low-jitter external clock generator (CDC6CE025) is located in close physical proximity to the FPGA. This minimises trace length and parasitic effects, resulting in improved clock integrity and reduced phase noise for internal PLLs and timing domains within the device.

Series source termination resistors (SSR) on high-speed paths have been intentionally omitted. Due to the extremely short trace lengths and tight component placement, signal integrity is inherently maintained without the need for additional series damping, reducing part count and complexity without compromising performance.

The JTAG and configuration strategy follows Microchip guidelines but is optimised for a permanently driven external interface (via FTDI in the Debug POD context). As such, certain recommended pull-down resistors are omitted because all signals are actively driven, ensuring defined logic levels at all times.

The JTAG and configuration strategy follows Microchip guidelines but is optimised for a permanently driven external interface (via FTDI in the Debug POD context). As such, certain recommended pull-down resistors are omitted because all signals are actively driven, ensuring defined logic levels at all times.

## Practical Implementation Notes (PCB Focused)

To ensure that the electrical performance, stability, and long-term reliability of the M2GL05 CORE & I/O CCB are fully realised in hardware, the following implementation practices shall be observed during PCB layout and assembly.

FPGA Placement and Orientation
The IGLOO2 (M2GL005-VFG256) device shall be positioned centrally within the PCB region allocated to the CCB. This minimises average trace length to all supply decoupling, clock sources, external memory devices, and interface connectors. Orientation should be selected such that high-activity pin banks and clock resources are physically close to their associated support circuitry.

Decoupling Strategy
Distributed 100 nF capacitors shall be placed as close as physically possible to each VDD and VDDIO pin group. Connections should be made with short, wide copper traces directly to the relevant power pins, minimising loop area. In addition to local high-frequency decoupling, bulk capacitors (10 µF and above) shall be placed on the P1V2 core supply to support transient current demands associated with internal FPGA switching activity.

PLL and Analogue Supply Filtering
The PLL analogue supply (VDDA) shall be isolated from the main P3V3 rail using a ferrite bead (BLM15AG601SN1D or equivalent), followed by a local decoupling network consisting of both bulk and high-frequency MLCCs. This provides high-frequency isolation and reduces jitter sensitivity of the internal clocking network.

Clock Generator Placement
The CDC6CE025 clock generator and its associated decoupling network shall be located in very close proximity to the designated clock input pins of the FPGA. Clock traces should be kept short, direct, and free from aggressive via usage. Where possible, reference planes under clock traces should be continuous and impedance-stable to minimise phase noise and reflection.

External SRAM Placement
If the optional IS66WVQ8M4DBLL-133BLI SRAM is fitted, it shall be placed on the same side of the PCB as the FPGA and in close physical proximity. All data and clock traces between the FPGA and SRAM should be matched in length where practical, with particular care taken on the SCLK and DQSM lines. No series source resistors are required due to short trace lengths and controlled routing.

JTAG and Configuration Signals
JTAG, reset and configuration signals (JTAG_TCK, TMS, TDI, TDO, TRST_B, FLASH_GOLDEN_N, DEVRST_N, BOR_B and JTAGSEL) shall be routed as short point-to-point connections to the controlling device or header. These traces are assumed to be driven by an external FTDI device in normal use; therefore, termination and pull-resistor strategies may be simplified provided that driven states are guaranteed.

Grounding and Return Paths
A solid, low-impedance ground plane shall be provided beneath the FPGA and its supporting circuitry. High-speed digital traces should always be routed with an uninterrupted return path directly beneath in an adjacent plane layer to minimise loop area and EMI. Avoid splitting reference planes beneath critical clock or memory buses.

Power Distribution
Power rails (P3V3, P2V5, P1V2) should be routed using wide copper pours where possible, with local point-of-load decoupling applied near consuming devices. Avoid daisy-chaining supplies between devices when star-style distribution can be achieved.

Thermal Considerations
While the M2GL005 is a low-power device, a solid copper region beneath the BGA and multiple thermal vias to internal planes are recommended to improve heat spreading and to support long-term reliability, especially when operating at elevated ambient temperatures.

Silkscreen and Documentation
Clear silkscreen labelling of supply rails, key signals (clock, reset, JTAG) and pin-1 orientation is strongly recommended. This aids in bring-up, troubleshooting, and long-term maintainability of the board.

## Test / Bring-Up Procedure

The following procedure defines a controlled, repeatable method for safe initial power-up, verification, and functional validation of the M2GL05 CORE & I/O CCB. Each step shall be completed in sequence. Any abnormal result must be investigated and resolved before proceeding.

1. Pre-Power Inspection
Perform a detailed visual inspection of the PCB under magnification. Confirm correct orientation and placement of the FPGA, clock generator, SRAM (if fitted), capacitors, resistors, and ferrite beads. Inspect for solder bridges, open joints, damaged pads, or contamination. Verify that no unintended copper bridges exist between independent power nets.

2. Ground Continuity Verification
Using a digital multi-meter, check for solid continuity across the main ground plane (0V). Verify that no short circuits exist between 0V and any of the power rails (P3V3, P2V5, P1V2, VDDA, VREF or VPP rails).

3. Rail Short Testing (Unpowered)
Measure the resistance of each supply rail to ground. While exact values will vary, all rails should exhibit a non-zero resistance and should not appear as a direct short (≈0 Ω). Any suspiciously low resistance should be investigated.

4. Initial Power Application (Current-Limited)
Apply power to the board from a current-limited laboratory supply set to the intended input voltage for the regulators supplying P3V3, P2V5 and P1V2. Configure the current limit conservatively (e.g. 100–200 mA) for the initial test. Monitor supply current for abnormal draw.

5. Voltage Rail Verification
Measure all primary supply rails:

- P3V3

- P2V5 (if populated)

- P1V2 (core)

- VDDA / VSSA (PLL supply)

Confirm that all values are within expected tolerances and remain stable. Excessive ripple or oscillation should be investigated before proceeding.

6. Clock Verification
Using an oscilloscope or frequency counter (where accessible), verify the presence, frequency and stability of the clock signal provided by the CDC6CE025. Confirm that it is correctly presented at the FPGA clock input pin and is free from visible distortion or excessive noise.

7. JTAG Connectivity Check
Connect the JTAG interface from the Debug POD / FTDI interface to the board. Confirm that the device is detected by the programming toolchain. A correct IDCODE read indicates functional power, clock and JTAG connectivity, specifically under Libero use the FlashPro tools to validate the JTAG chain.

8. Configuration Test
Attempt to load a simple test bitstream or design (e.g. LED toggle, I/O toggle, or heartbeat design) into the IGLOO2 device. Confirm that the device programs successfully and that expected basic behaviour is observed.

9. SRAM (if fitted) Verification
If the optional IS66WVQ8M4DBLL SRAM is populated, perform a basic read/write test via the FPGA design. Confirm correct data retention and read-back across multiple addresses.

10. Soak and Stability Test
Allow the board to operate for a minimum of 30 minutes. Monitor rail stability and observe device temperature via touch probe or thermal camera. It should remain within normal operating limits. Any sign of thermal runaway or instability must be addressed.

11. Final Acceptance
Record key measurements, configuration success, and serial identification (if applicable). Once all tests pass, the CCB may be approved for integration into higher-level assemblies or test systems.

## Electrical Specification Summary

| Parameter | Typical / Design Value | Notes |
| --- | --- | --- |
| FPGA Device | M2GL005-VFG256I (IGLOO2) | Flash-based, non-volatile, low-power |
| Core voltage (P1V2) | 1.2 V nominal | FPGA core supply |
| Auxiliary voltage (P2V5) | 2.5 V nominal | Bank / support functions |
| I/O voltage (P3V3) | 3.3 V nominal | Default I/O supply |
| PLL analogue supply | Filtered from P3V3 | Via BLM15AG601SN1D ferrite bead |
| Clock source | CDC6CE025 | Low-jitter programmable clock generator 25 MHz |
| External memory | IS66WVQ8M4DBLL-133BLI | 8 Mb (1 MB) SRAM, optional |
| Configuration method | JTAG | SPI unused in this design |
| General-purpose I/O | All unassigned | User-defined pin mapping |
| Supported I/O banks | DDRIO / MSIO banks | Multi-standard capable |
| Reset / control | BOR_B, DEVRST_N, TRST_B | JTAG & system control |
| Operating temperature (intent) | -40 °C to +85 °C (target) | Device-dependent rating |
| Design lifetime (intent) | 25 years | Long-term reliability goal |
| Isolation | Noninherent | Provided by upstream CCBs |

The electrical characteristics of this CCB are primarily governed by the M2GL005 IGLOO2 FPGA and its supporting voltage regulators, clock source, and memory devices. All values listed are nominal design targets and must be verified within the context of the complete system implementation, PCB layout, and external power architecture.

This CCB does not provide an isolated power domain; it is intended to be powered from a well-regulated, already-isolated or system-referenced supply when used in high-voltage environments.

Component Function and Design Rationale Table

| Designator(s) | Description of function | Required? | Rationale / Notes |
| --- | --- | --- | --- |
| U1A / U1B – M2GL005-VFG256I | Core IGLOO2 FPGA device providing all programmable logic, timing and I/O resources | Yes | Central digital processing element. Flash-based, non-volatile configuration, deterministic start-up and good radiation tolerance. All general-purpose I/O deliberately left uncommitted to maximise reuse across applications. |
| X1 – CDC6CE025000ADLER | 3.3 V low-jitter programmable clock generator | Yes | Provides the primary system clock into dedicated FPGA clock pins. Placed in very close proximity to the FPGA so no series source resistors are required on the clock path. On-chip PLLs rely on this low-jitter reference for stable timing. |
| FB1 – BLM15AG601SN1D | Ferrite bead in series with VDDA (PLL analogue supply) | Yes | Forms part of the dedicated PLL supply filter network between P3V3 and VDDA. Attenuates high-frequency noise and isolates digital rail transients from sensitive analogue PLL circuitry, improving jitter performance. |
| C16–C19 (100 nF) and associated VDDA bulk caps | Local decoupling for VDDA / VSSA (PLL supply) | Yes | Provide high-frequency and bulk decoupling on the filtered PLL rail, in combination with FB1. Reduce supply noise and spur coupling into PLLs, directly supporting clock stability. |
| C1–C5, C8–C15 (100 nF MLCC) | Distributed high-frequency decoupling on P3V3 and P2V5 | Yes | Placed close to FPGA and SRAM supply pins to minimise local supply impedance and suppress high-frequency switching noise on the 3.3 V and 2.5 V rails. Form the primary HF bypass network. |
| C6, C7, C20–C23 (10 µF MLCC) | Bulk capacitance on P3V3 and P1V2 rails | Yes | Provide local energy storage for the core (P1V2) and I/O (P3V3) rails, supporting dynamic load changes and reducing low-frequency ripple. Total P1V2 bulk is 40 µF as per design notes. |
| U3 – TPSM828438VCFR | 3 A step-down power module (P5V to P2V5) | Yes | Generates regulated 2.50 V I/O supply for IGLOO2 banks. Configured using RSET = 44.2 kΩ (1 %) in accordance with TI table. Provides ±1 % accuracy, low noise and high efficiency. |
| U4 – TPSM828437VCFR | 3 A step-down power module (P5V to P1V2) | Yes | Generates regulated 1.20 V core supply for IGLOO2. Configured using RSET = 56.2 kΩ (1 %) per TI datasheet. Optimised for low ripple and fast transient response. |
| R4 – 44.2 kΩ (1 %, 0402) | VSET resistor for TPSM828438 | Yes | Selects 2.50 V output voltage from TPSM828438 module based on TI VSET table. |
| R5 – 56.2 kΩ (1 %, 0402) | VSET resistor for TPSM828437 | Yes | Selects 1.20 V output voltage from TPSM828437 module based on TI VSET table. |
| C24, C26 – 4.7 µF X5R | Input decoupling for TPSM82843x VIN | Yes | Required local input reservoir for stable operation from 5 V rail. |
| C25, C27 – 10 µF X7R | Output bulk capacitance for TPSM82843x | Yes | Provide low ESR output filtering and transient current buffering. |
| U2 – IS66WVQ8M4DBLL-133BLI | 8-Mb (1 MB) external SRAM, QSPI-style interface | Optional | Extends available memory for buffering, trace capture and debug data without consuming internal FPGA RAM. Located very close to the FPGA; short, controlled-length traces allow omission of series resistors while maintaining signal integrity. |
| C2–C4 (100 nF) near U2 | Local decoupling for external SRAM | Optional | Reduce supply impedance at the SRAM package and limit supply bounce during high-speed read/write operation. Required if U2 is fitted. |
| R1, R2 – 1 kΩ (0402) | Pull-up resistors from JTAG_TCK / JTAG_TMS (and/or related lines) to P3V3 | Yes | Provide defined logic levels on key JTAG/control lines when the driving FTDI device is inactive. Value chosen to be strong enough for noise immunity while not excessively loading the interface. |
| R3 – 10 kΩ (0402) | Bias resistor on FPGA_BOR_B (brown-out reset) | Yes | Ensures the BOR_B input assumes a defined state in the absence of external drive, in line with Microchip recommendations, and supports controlled brown-out behaviour for deterministic reset. |
| JTAG_TCK, TMS, TDI, TDO, TRST_B, JTAGSEL, FLASH_GOLDEN_N, DEVRST_N nets | Configuration, reset and debug control signals | Yes | Provide full JTAG configuration and control access to the IGLOO2 device. Routed short and directly to the FTDI interface / header. SPI configuration path is intentionally unused in this CCB to reduce complexity. |
| MSIO / DDRIO bank pins | General-purpose, multi-standard I/O resources | Application-dependent | All MSIO/DDRIO pins are brought out but left uncommitted, to be assigned at system level (protocols, interfaces, debug links). This preserves maximum flexibility and allows the same CCB to be re-used across multiple projects. |

All components have been selected to balance performance, reliability and part-count efficiency. Close physical coupling of U1, X1 and U2 intentionally allows the omission of series source resistors on high-speed nets, relying instead on very short, well-controlled traces to maintain signal integrity while reducing component count and potential failure sites.

Notes on Part Selection Philosophy

Preference for long-lifecycle, industry-standard parts from established suppliers

Use of X7R/X5R MLCCs with conservative voltage derating

Minimised number of different component values to simplify stocking and assembly

Design intent for 25-year operational life in controlled industrial / lab environments

Preliminary FIT Rate Estimate (Structure based on FIDES Methodology)

A high-level reliability projection for the M2GL05 CORE & I/O CCB can be established using a FIDES-style component-based failure rate model. The intent of this estimate is not to provide a guaranteed lifetime, but to support comparative design decisions, derating strategies, and long-life suitability analysis.

| Component Group | Typical Qty (this CCB) | Primary Function | Estimated FIT Contribution |
| --- | --- | --- | --- |
| M2GL005 FPGA | 1 | Core processing and I/O | 25.0 |
| CDC6CE025 Clock Generator | 1 | System timing / PLL reference | 6.0 |
| TPSM828438VCFR (P5V → P2V5) | 1 | 2.5 V I/O rail regulator | 8.0 |
| TPSM828437VCFR (P5V → P1V2) | 1 | 1.2 V core rail regulator | 8.0 |
| IS66WVQ8M4 SRAM (optional) | 1 | External buffer memory | 12.0 |
| MLCC capacitors (100 nF & 10 µF) | 23 | Decoupling / bulk storage | 3.0 |
| Ferrite bead (BLM15AG601SN1D) | 1 | Supply filtering (PLL rail) | 1.5 |
| Resistors (0402) | 3 | Bias / pull-up / pull-down | 0.06 |

Estimated Total FIT (if SRAM fitted): ~63.6 failures / 10⁹ hours
Equivalent failure rate (λ): ≈ 6.36 × 10⁻⁸ / hour
Estimated MTBF: ≈ 1.6 × 10⁷ hours (≈ 1,800 years)

These values are order-of-magnitude engineering estimates derived using the FIDES methodology structure (separate physical, part-manufacturing and process contributions) but do not replace a full, tool-based FIDES calculation using an agreed mission profile and audited process factors. They are intended for early-phase comparative analysis only.

Using a simple exponential failure model, the projected notional reliability is:
– Survival at 10 years (≈ 87,600 h): ≈ 99.4 %
– Survival at 25 years (≈ 219,000 h): ≈ 98.6 %

Actual field reliability will depend strongly on:
– The true mission profile (temperature, vibration, humidity, power cycling)
– PCB technology, assembly quality and inspection levels
– Mechanical environment (vibration, shock) and enclosure design
– Contamination control, conformal coating, and maintenance practice

For any formal programme- or customer-level reliability commitment, the integrator shall perform a full FIDES analysis using the complete BOM, agreed mission profile and process audit, and then substitute the resulting FIT/MTBF values into this section.

Note: Final system FIT and MTBF shall be determined by the integrator, using full BOM data, actual operating profile, and an appropriate standard (e.g. FIDES, MIL-HDBK-217, IEC TR 62380).

Full licence text: CERN Open Hardware Licence Version 2 – Strongly Reciprocal (CERN-OHL-S v2), CERN